SIMATS ENGINEERING

ASSESSMENT TOOL 2

| Course Name | Computer Networks |
| --- | --- |
| Course Code | CSA07 |
| Course Outcome | CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3) |
| Assessment Weightage | 30% |
| Questions / Total Marks | 5 Questions ¦ 50 Marks |
| Annexure | Annexure A — Scenario-Based |

ANSWER BOOKLET

Annexure A — Scenario-Based

Code of Conduct

I Jotheesh.A(Name / Reg No: 192511029) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: A.Jotheesh

Date:  10.08.2026

This booklet contains detailed, step-by-step solutions to all five scenario-based questions — including worked derivations, verification cross-checks, common-mistake notes and engineering context for each — followed by the evaluation rubric on the final page.

Contents

Question 1 — Smart Campus VLSM Allocation — 192.168.100.0/24

Question 2 — Multinational Company VLSM Allocation — 10.10.0.0/16

Question 3 — Smart Hospital IPv6 Migration — /64 Subnet

Question 4 — ISP Core Router — Packet Forwarding & Routing Table Lookup

Question 5 — Two-Department LAN Design via Layer-3 Router

Rubrics for Calculation

Quick Reference — Key Formulas & Rules Used in This Booklet

Every calculation in this booklet reduces to one of the relationships below. Keeping them together here makes it easy to double-check any step without hunting back through the worked solutions.

| Quantity / Rule | Formula | Notes |
| --- | --- | --- |
| Block size (IPv4) | 2^(32 − prefix) | Total addresses in one subnet, including network & broadcast |
| Usable hosts (IPv4) | 2^(32 − prefix) − 2 | Subtracts the reserved network ID and broadcast ID |
| IPv6 hosts per prefix | 2^(128 − prefix) | Same idea as IPv4, scaled to a 128-bit address |
| Subnet membership | floor(address ÷ block size) × block size | Gives the network address that owns a given host address |
| Longest Prefix Match | Router picks the routing-table entry with the most specific (longest) matching prefix | Governs how a router chooses among several candidate routes |
| Gateway convention | First usable address in the subnet (network address + 1) | Common (not mandatory) practice for assigning a LAN's default gateway |

Question 1  —  Smart Campus VLSM Allocation — 192.168.100.0/24

Scenario: A newly established Smart Campus is deploying a secure network for its academic and administrative departments. The administrator has been allocated 192.168.100.0/24. The following masks have already been assigned: School of Computing → /25, School of Electronics → /26, School of Mechanical Engineering → /27.

Concept Overview

Each department has a different mask because each has a different expected device count — this is Variable Length Subnet Masking (VLSM). The task is to carve the single /24 block into three correctly sized, non-overlapping pieces, largest requirement first, and then read off the standard four figures (network, broadcast, host range, usable hosts) for each.

Step 1: Block size for each department

Block size = 2^(32 − prefix length)

- School of Computing (/25) = 2^7 = 128 addresses

- School of Electronics (/26) = 2^6 = 64 addresses

- School of Mechanical Engineering (/27) = 2^5 = 32 addresses

Step 2: Allocate sequentially from 192.168.100.0 (largest block first)

- Computing: 192.168.100.0 – 192.168.100.127 (128 addresses)

- Electronics: 192.168.100.128 – 192.168.100.191 (64 addresses)

- Mechanical: 192.168.100.192 – 192.168.100.223 (32 addresses)

Step 3: Network address, broadcast address, valid host range, usable hosts (Tasks 1–3)

| Usable hosts = 2^(32 − prefix) − 2 (2 addresses reserved for Network ID and Broadcast ID) |
| --- |
| Department | Subnet (CIDR) | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| Computing | 192.168.100.0/25 | 192.168.100.0 | 192.168.100.1 – 192.168.100.126 | 192.168.100.127 | 126 |
| Electronics | 192.168.100.128/26 | 192.168.100.128 | 192.168.100.129 – 192.168.100.190 | 192.168.100.191 | 62 |
| Mechanical | 192.168.100.192/27 | 192.168.100.192 | 192.168.100.193 – 192.168.100.222 | 192.168.100.223 | 30 |

Step 4: Remaining unused IP address range (Task 4)

The full /24 block spans 192.168.100.0 – 192.168.100.255 (256 addresses). After allocating all three departments, addresses 192.168.100.0 – 192.168.100.223 (224 addresses) are used.

Unused range = 192.168.100.224 – 192.168.100.255 (256 − 224 = 32 addresses remain available)

Verification

Each department's starting address (0, 128, 192) is an exact multiple of that department's own block size, confirming correct alignment; the blocks are contiguous (Computing ends at .127, Electronics starts at .128; Electronics ends at .191, Mechanical starts at .192), so no address is skipped or double-counted.

Common Mistakes to Avoid

- Forgetting that /25 gives 126 usable hosts, not 128 — two addresses are always reserved.

- Allocating the departments in the order listed in the scenario rather than largest-block-first, which can misalign later boundaries.

- Reporting the unused range as a separate /27 by habit — here it happens to be 32 addresses, but that is simply what is left over, not a mask the campus has chosen.

Engineering Relevance

The leftover 32 addresses (192.168.100.224–255) give the campus room to add a fourth department, a management/Wi-Fi-controller VLAN, or point-to-point router links later without renumbering any of the three schools already deployed — exactly the kind of headroom a real campus network design should keep in reserve.

Final Answer Computing: Network 192.168.100.0/25, Hosts 192.168.100.1–192.168.100.126, Broadcast 192.168.100.127, 126 usable hosts. Electronics: Network 192.168.100.128/26, Hosts 192.168.100.129–192.168.100.190, Broadcast 192.168.100.191, 62 usable hosts. Mechanical: Network 192.168.100.192/27, Hosts 192.168.100.193–192.168.100.222, Broadcast 192.168.100.223, 30 usable hosts. Unused range remaining in the /24 block: 192.168.100.224 – 192.168.100.255 (32 addresses).

Question 2  —  Multinational Company VLSM Allocation — 10.10.0.0/16

Scenario: A multinational software company is expanding its headquarters and has been assigned 10.10.0.0/16. The administrator has allocated: Software Development → /24, Quality Assurance → /25, Human Resources → /26, Executive Management → /27.

Concept Overview

The method is the same VLSM process as Question 1, scaled up to a /16 parent block (65,536 addresses). With four departments of decreasing size, the block boundaries roll over from the third octet into the fourth octet partway through the allocation — a detail worth tracking carefully.

Step 1: Block size for each department

Block size = 2^(32 − prefix length)

- Software Development (/24) = 2^8 = 256 addresses

- Quality Assurance (/25) = 2^7 = 128 addresses

- Human Resources (/26) = 2^6 = 64 addresses

- Executive Management (/27) = 2^5 = 32 addresses

Step 2: Allocate sequentially from 10.10.0.0 (largest block first)

- Software Development: 10.10.0.0 – 10.10.0.255 (256 addresses — the entire first /24 of the block)

- Quality Assurance: 10.10.1.0 – 10.10.1.127 (128 addresses)

- Human Resources: 10.10.1.128 – 10.10.1.191 (64 addresses)

- Executive Management: 10.10.1.192 – 10.10.1.223 (32 addresses)

Note how Software Development's 256-address block consumes the whole 10.10.0.x range by itself, so every subsequent department's addresses start counting from 10.10.1.x instead.

Step 3: Network address, broadcast address, valid host range, usable hosts (Tasks 1–3)

| Department | Subnet (CIDR) | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Software Dev. | 10.10.0.0/24 | 10.10.0.0 | 10.10.0.1 – 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assur. | 10.10.1.0/25 | 10.10.1.0 | 10.10.1.1 – 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | 10.10.1.128/26 | 10.10.1.128 | 10.10.1.129 – 10.10.1.190 | 10.10.1.191 | 62 |
| Exec. Mgmt. | 10.10.1.192/27 | 10.10.1.192 | 10.10.1.193 – 10.10.1.222 | 10.10.1.223 | 30 |

Step 4: Remaining unused IP address range (Task 4)

The full /16 block spans 10.10.0.0 – 10.10.255.255 (65,536 addresses). After allocating all four departments, addresses 10.10.0.0 – 10.10.1.223 (256 + 128 + 64 + 32 = 480 addresses) are used.

Unused range = 10.10.1.224 – 10.10.255.255 (65,536 − 480 = 65,056 addresses remain available)

Verification

Each block's start (0 in the 3rd octet for Software Dev.; then 0, 128, 192 in the 4th octet for QA, HR and Executive) is a multiple of its own block size, and each block ends exactly where the next begins — confirming the plan is contiguous and correctly aligned.

Common Mistakes to Avoid

- Assuming every department's addresses start fresh at 10.10.X.0 — once Software Development consumes the full first /24, later departments continue counting within 10.10.1.x, not a new octet each time.

- Forgetting that a /16's 'remaining space' calculation must subtract from the full 65,536-address block, not just from one /24 slice of it.

Engineering Relevance

Reserving an entire /24 for Software Development but only smaller blocks for HR and Executive Management mirrors real headcount ratios at a software company — engineering typically dwarfs back-office staff in device count. The 65,056 unused addresses give the company enormous room to add new departments, regional offices, or a VPN/VoIP address pool without ever renumbering the four ranges already assigned.

Final Answer Software Development: Network 10.10.0.0/24, Hosts 10.10.0.1–10.10.0.254, Broadcast 10.10.0.255, 254 usable hosts. Quality Assurance: Network 10.10.1.0/25, Hosts 10.10.1.1–10.10.1.126, Broadcast 10.10.1.127, 126 usable hosts. Human Resources: Network 10.10.1.128/26, Hosts 10.10.1.129–10.10.1.190, Broadcast 10.10.1.191, 62 usable hosts. Executive Management: Network 10.10.1.192/27, Hosts 10.10.1.193–10.10.1.222, Broadcast 10.10.1.223, 30 usable hosts. Unused range remaining in the /16 block: 10.10.1.224 – 10.10.255.255 (65,056 addresses).

Question 3  —  Smart Hospital IPv6 Migration — /64 Subnet

Scenario: A smart hospital is migrating from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. Assigned network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64.

Concept Overview

This address is given already in its network form — every host bit is zero — so the tasks are the same compression/expansion/sizing skills used for IPv6 elsewhere in this course, applied to a genuine access-network scenario: a hospital's medical-device network is exactly the kind of large, IoT-heavy deployment IPv6's enormous address space was designed for.

Task 1: Compress the address to its shortest form

Rule 1 — remove leading zeros within each 16-bit group:

2001 : db8 : abcd : 0 : 0 : 0 : 0 : 0

Rule 2 — replace the longest run of consecutive all-zero groups with a single '::': the last five groups (4th through 8th) are all zero and form the longest such run, so they compress together:

Compressed form: 2001:db8:abcd::

Task 2: Expand the compressed address back to full notation

Expanding 2001:db8:abcd::, the '::' is replaced with enough all-zero groups to restore all 8 groups (3 groups already present, so 5 zero groups are inserted), and every group is padded back to 4 hex digits:

Expanded form: 2001:0db8:abcd:0000:0000:0000:0000:0000

Task 3: Network prefix for /64

A /64 prefix keeps the first 64 bits (the first four groups) as the network portion:

Network prefix: 2001:0db8:abcd:0000::/64 (compressed: 2001:db8:abcd::/64)

Tasks 4 & 5: Total number of host addresses supported

Tasks 4 and 5 both ask for the same quantity — the number of unique interface identifiers obtainable from the 64 host bits left after the /64 prefix — so a single calculation answers both:

Total host addresses = 2^(128 − 64) = 2^64 = 18,446,744,073,709,551,616

Putting 2^64 in Perspective

This single hospital subnet can address more devices than there are IPv4 addresses in existence, repeated over four billion times (2^64 ÷ 2^32 = 2^32 ≈ 4.29 billion). In practice the hospital will never come close to exhausting a /64 — the huge address space is allocated per subnet by design, to keep IPv6 addressing hierarchical and simple rather than to conserve addresses the way IPv4 subnetting must.

Common Mistakes to Avoid

- Compressing only part of the zero run, e.g. writing 2001:db8:abcd:0:0:0:0:0 with '::' in the wrong place instead of collapsing all five trailing zero groups together.

- Treating Task 4 and Task 5 as requiring two different formulas — both describe the same 64 host bits, so both have the same numeric answer.

- Dropping 'abcd' or truncating it — unlike an all-zero group, a non-zero group is never removed, only leading zeros within it are.

Engineering Relevance

Allocating one /64 to the hospital's medical-device network means every infusion pump, monitor, and IoT sensor can receive a unique, stable IPv6 address (commonly via SLAAC) without the address-exhaustion pressure that plags large IPv4 IoT deployments — an important reliability consideration in a clinical setting where device connectivity is safety-critical.

Final Answer Compressed address: 2001:db8:abcd:: Expanded address: 2001:0db8:abcd:0000:0000:0000:0000:0000 Network prefix (/64): 2001:db8:abcd::/64 (i.e., 2001:0db8:abcd:0000::/64) Total possible host addresses within this /64 network (Tasks 4 & 5): 2^64 = 18,446,744,073,709,551,616

Question 4  —  ISP Core Router — Packet Forwarding & Routing Table Lookup

Scenario: An ISP connects multiple customer networks through a core router. Routing table entries: 192.168.1.0/24, 192.168.2.0/24, 192.168.3.0/24. A packet arrives with destination IP 192.168.2.45.

Concept Overview — How a Router Chooses a Route

A router compares a packet's destination address against every entry in its routing table and forwards using the entry whose network best matches — formally, the Longest Prefix Match (LPM) rule: among all matching entries, the one with the longest (most specific) prefix wins. Here all three entries are /24s covering non-overlapping ranges, so at most one of them can match any given address, making the lookup a simple containment check rather than a genuine LPM tie-break.

Task 1: Identify the destination subnet

Each /24 entry covers exactly 256 addresses (its own third octet):

- 192.168.1.0/24 → 192.168.1.0 – 192.168.1.255

- 192.168.2.0/24 → 192.168.2.0 – 192.168.2.255

- 192.168.3.0/24 → 192.168.3.0 – 192.168.3.255

The destination 192.168.2.45 has third octet 2 and last octet 45 (0 ≤ 45 ≤ 255), so it falls inside 192.168.2.0 – 192.168.2.255 → destination subnet = 192.168.2.0/24.

Task 2: Matching routing table entry

Matching entry: 192.168.2.0/24

This is the only entry whose range contains 192.168.2.45, so no further Longest-Prefix-Match tie-break between multiple candidates is needed in this particular case.

Task 3: Next-hop network / interface

Because 192.168.2.0/24 appears in the routing table as a directly connected customer network (not learned via a routing protocol pointing to another router), the 'next hop' is simply the router's own local interface configured with an address inside 192.168.2.0/24. The router forwards the packet out that interface and delivers it to 192.168.2.45 directly at Layer 2 (resolving the host's MAC address via ARP/Neighbor Discovery) — no further router hop is required.

Task 4: Default route behaviour if no entry matches

If a destination address does not fall inside any of the three /24 ranges above, the router consults its default route (commonly written 0.0.0.0/0), which typically points toward the ISP's upstream backbone connection — the 'gateway of last resort.' If no default route is configured either, the router has no way to forward the packet and instead drops it, returning an ICMP 'Destination Unreachable' message to the sender.

Default route: 0.0.0.0/0 → forwarded to the ISP's upstream/backbone link (gateway of last resort)

Common Mistakes to Avoid

- Confusing 'next hop' with 'destination host' — for a directly connected network, the next hop is the router's own interface, not another router's IP address.

- Assuming Longest Prefix Match requires comparing prefix lengths here — it only becomes decisive when two or more entries could both match the same address (e.g. a /24 and a covering /16); with three disjoint /24s, only one entry can ever match.

- Forgetting that a default route only activates when no more specific entry matches — it is always the least-preferred (least specific, /0) route in the table.

Engineering Relevance

This is exactly how an ISP's edge/core router forwards traffic to directly attached customer LANs while still being able to reach the wider Internet: specific customer routes take priority via Longest Prefix Match, and anything unrecognised falls back to the default route toward the ISP's core network.

Final Answer Destination subnet: 192.168.2.0/24 (192.168.2.45 falls within 192.168.2.0–192.168.2.255). Matching routing table entry: 192.168.2.0/24. Next-hop / interface: the router's own interface directly connected to 192.168.2.0/24 — the packet is delivered directly to the host, no further router hop needed. If no entry matches: the router forwards via its default route (0.0.0.0/0) toward the ISP's upstream link; if none exists, the packet is dropped with an ICMP Destination Unreachable reply.

Question 5  —  Two-Department LAN Design via Layer-3 Router

Scenario: A company establishes two departments connected through a Layer-3 router. Administration LAN → 192.168.10.0/26, Finance LAN → 192.168.10.64/26. Router interfaces: Administration Gateway → 192.168.10.1, Finance Gateway → 192.168.10.65.

Concept Overview

Both LANs use a /26 mask (block size 64), and are already laid out back-to-back starting at 192.168.10.0. A Layer-3 router with one interface (and IP address) in each subnet lets hosts in Administration and Finance reach each other only via the router — they are on different broadcast domains even though they share the same parent /24.

Task 1: Network address and broadcast address for each LAN

| Block size for /26 = 2^(32−26) = 64 addresses |
| --- |
| LAN | Subnet (CIDR) | Network Address | Broadcast Address |
| Administration | 192.168.10.0/26 | 192.168.10.0 | 192.168.10.63 |
| Finance | 192.168.10.64/26 | 192.168.10.64 | 192.168.10.127 |

Task 2: Valid host IP address range for each LAN

| Usable hosts = 2^(32−26) − 2 = 64 − 2 = 62 per LAN |
| --- |
| LAN | Valid Host Range | Usable Hosts |
| Administration | 192.168.10.1 – 192.168.10.62 | 62 |
| Finance | 192.168.10.65 – 192.168.10.126 | 62 |

Both gateway addresses given in the scenario (.1 and .65) fall correctly at the start of each LAN's valid host range, following the common convention of assigning the first usable address in a subnet to the router/gateway interface.

Task 3: Suggested IP addresses for three computers in each LAN

Any three addresses from each valid host range, excluding the gateway address already in use by the router, are acceptable. Reasonable, easy-to-track choices are the next three addresses after the gateway:

| LAN | PC-1 | PC-2 | PC-3 |
| --- | --- | --- | --- |
| Administration | 192.168.10.2 | 192.168.10.3 | 192.168.10.4 |
| Finance | 192.168.10.66 | 192.168.10.67 | 192.168.10.68 |

Task 4: Default gateway for hosts in each LAN

- Administration LAN hosts (192.168.10.1 – 192.168.10.62) → default gateway 192.168.10.1

- Finance LAN hosts (192.168.10.65 – 192.168.10.126) → default gateway 192.168.10.65

Each host must point to the router interface configured on its own subnet — a host cannot use a gateway address that lies outside its own valid host range, since that address would be unreachable without already going through a router.

Common Mistakes to Avoid

- Assigning a suggested PC address that collides with the gateway address (.1 or .65) already in use by the router interface.

- Mixing up the two gateways — Finance hosts must use 192.168.10.65, not 192.168.10.1, even though both belong to the same company and the same parent /24.

- Forgetting that .63 and .127 are broadcast addresses and .0/.64 are network addresses — none of the four are assignable to a PC or router interface.

Engineering Relevance

Splitting Administration and Finance into separate /26 broadcast domains, joined only through the Layer-3 router, is standard practice for departmental segmentation: it limits broadcast traffic to each department, lets the router apply access-control policies between the two (e.g., restricting Administration from reaching Finance's payroll systems), and keeps each department's addressing simple and self-contained.

Final Answer Administration LAN: Network 192.168.10.0/26, Broadcast 192.168.10.63, Hosts 192.168.10.1–192.168.10.62 (62 usable), Gateway 192.168.10.1. Finance LAN: Network 192.168.10.64/26, Broadcast 192.168.10.127, Hosts 192.168.10.65–192.168.10.126 (62 usable), Gateway 192.168.10.65. Suggested PCs — Administration: .2, .3, .4 ¦ Finance: .66, .67, .68. Default gateway — Administration hosts use 192.168.10.1; Finance hosts use 192.168.10.65.

RUBRICS FOR CALCULATION

Scenario Based Assessment

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

Total Weightage: 100%   |   Total Marks: 50